Parallel Communication Patterns

Map - one to one correspondence between input and output

Gather(many to one) - Gather data from multiple locations and then write it to one memory location(convolution)

Scatter(ont to many) - Data form a single location is written to multiple locations

(Several threads attempt to write at the same place at the same time) - may cause problems

Stencil(several to one) - Date Reuse - Many Thread accessing and using the same data

Transpose - Reorder Data elements in Memory

Reduce

Sort/Scan

Sharing Memory//Safely - GPU Hardware -

Summary of Programming Model -

Kernels - C/C++ function - Performed by many threads

Each kernel may have different number of threads per thread blocks

Thread Blocks and GPU Hardware -

- CUDA GPU - Streaming Multiprocessors (SM)

- SM -> Simple Processors/Memory

- GPU is responsible for allocating blocks to SMs

- All the SMs run in parallel and independently.

- A SM may run multiple blocks but A block cannot be run on multiple SMs

- Thread in different thread blocks must not cooperate with each other.

- Threads in a single thread block may cooperate with each other

- Programmer cannot specify any order of execution for the thread blocks.

- No assumptions about what block will run on which SM

- No communication between blocks - “DeadLocks”

- // force the printf()s to flush

- cudaDeviceSynchronize();

When and Where - CUDA gaurantees -

- Al threads in a block run on the same SM at the same time

- All blocks in a kernel finish before any blocks from the next kernel run

Memory Model -

- Local Memory - local variables/parameters

- Shared Memory

- Global Memory

Synchronization -

Threads need to synchronize.

Barrier - Point in the program where all the threads stop and wait for others, and when all the threads have arrived at the barrier than the move further.

Use __syncthreads() to put barrier - sync threads within a block.. Why?

CUDA a hierarchy of

- Computation

- Memory

Minimize time spent on memory -

- Move frequently-accessed data to fast memory

Local > shared >> global >>CPU(host)    		(In terms of speed)

Local - In register or in L2 cache

Convention - h_ -> host

D_ -> device

- Coalesce global memory accesses

  - GPU most efficient when threads read or write contiguous memory locations.

Atomics - Read Guide for different atomic functions atomicAdd(ptr, val) for example.

Limitations -

- Only certain operations, data types(integers)

- Implement any atomic using CAS()

- Still no ordering constraints. (Non associativity of floating points)

- Serializes access to memory

- Atomics take time!

Thread Divergence -

When kernel have if statements / loops.

In loops some threads just sit around without doing anything.

Loop divergence slows the execution down.

Summary -